Outline

Title: A Depth-aware, Physics-based Augmentation Framework for Simulating Rain and Fog Distortions in Outdoor-RGB Images

Abstract

- Problem: rain and fog distortions in outdoor RGB images

- Method: depth-aware PBR simulation with Cycle-GAN refinement for rain; Cycle-GAN for fog augmentation

- Baseline dataset and training dataset: BDD100K; Data: clear-weather inputs, estimated depth

- Evaluation: downstream robustness plus visual quality

- Results

- Contribution: controllable adverse-weather augmentation pipeline

- Introduction (Draft is completed; no outline is needed for this part.)

  - Cycle-GAN and mechanism (Background)

- Related Work (Draft completed)

- Data Augmentation Methods

- Purpose for data augmentation

  - Rain Rendering

    - Physics-based rendering

- Rain particles are white-and-gray streaks directed in a specific direction. The appearance of raindrops is determined by its location relative to the camera.

- Depth map determines the size, colour and transparency of a rain particle: the closer a particle is to the camera, the larger it is in length; the closer a particle is to the camera, the dimmer (grayer) it is in colour; the further it is from the camera, the more transparent it is.

- The controllable intensity of rain particles is not uniform across the frame -> it’s based on the depth map; an area that is closer to the camera has less raindrops; vice versa.

- A filter that darkens the frame is added.

- Include examples from generated output.

    - Cycle-GAN

- Dataset: BDD100K. Labels are “clear”, “overcast”, “undefined”, “snowy”, “rainy”, “partly cloudy”, “foggy”; number of images in train split: 37344, 8700, 8119, 5549, 5070, 5881, 130, respectively; number of images in validation split: 5346, 1239, 1157, 769, 738, 738, 13, respectively.

- For training dataset, TrainA (clear, overcast and partly cloudy) and TrainB (rainy) both have size 3000. TestA and TestB both have size 400.

- Used PyTorch cycle-GAN model. Training parameters: learning rate: 0.0001; load size: 512; crop size: 384; batch size: 1; total number of epochs: 20 (10 epochs plus 10 decaying epochs).

- Include examples from training results

- The model darkens the frame and adds wet effect to the ground. Limitations: unstable; some images have random white noise. This issue mitigates when training size increases and image resolution increases.

    - Framework combining PBR and Cycle-GAN

- Augments 3000 clean images from BDD100K

- Include examples from outputs

  - Fog Rendering

- Use ACDC fog plus cityscapes datasets for training the GAN model

- Evaluation

  - Setup

- Downstream model: YOLOv8n

  - Evaluation 1: Comparing the performances of YOLOv8n on all BDD100K images (excluding foggy and snowy) from 5 training methods –clean, simple rain, depth rain, GAN, GAN+PBR

  - Evaluation 3: Comparing the performances of YOLOv8n on clean and fog ACDC images (excluding snow) by training using clean and fog-augmented dataset.

  - Comparing the performances of YOLOv8n on all BDD100K images (excluding snowy) from 2 training methods: all weather images vs. only GAN+PBR and Fog augmented

  - Analysis

- Augmentation methods help YOLOv8n to have better performances on object detection with the presence of natural distortions like rain and fog

- Result demonstrates the effectiveness of using data-augmentation as an alternative when a controlled dataset with distortion is not available

- Result demonstrates this framework to be feasible to increase object detection models’ ability to maintain a stable performance on images with natural distortions

- Application

- Training data expansion for outdoor RGB datasets; helps with the lack of controlled dataset

- Autonomous driving and outdoor object detection

- Weather stress testing under controllable severity

- Reusable augmentation framework for more datasets and potentially other distortions, such as snow, night, haze, etc.

- Limitation and improvement

- Dependence on depth map quality

- Physics-based rendering method is an estimation of real scenes-> lack of a full environment map means guesswork and estimation will primarily determine the appearance of raindrops

- GAN is not controllable; it could be more stable by increasing the training dataset size and resolution

- Future work: natural distortion augmentation on snow, night, haze, etc